MASSACHUSETTS INSTITUTE OF TECHNOLOGY

1.070J / 12.320J Introduction to Hydrology

Problem Set #6

Due November 13, 2008

(Include your Matlab m.files with the completed problem-set)

Problem 1:

Assume that for a location in the Amazon rain forest, diurnal variation in stomatal resistance is given by the data tabulated below. The stomatal resistance for any vegetation is mainly a function of time of day and soil moisture conditions.

| Local time | Stomatal resistance (s/cm) |
| --- | --- |
| 6:00 | 0.14 |
| 7:00 | 0.11 |
| 8:00 | 0.1 |
| 9:00 | 0.09 |
| 10:00 | 0.09 |
| 11:00 | 0.085 |
| 12:00 | 0.09 |
| 13:00 | 0.1 |
| 14:00 | 0.11 |
| 15:00 | 0.13 |
| 16:00 | 0.17 |
| 17:00 | 0.26 |
| 18:00 | 0.91 |

- using the tabulated data below for the same location, estimate the diurnal cycle for the atmospheric resistance. Graphically compare the atmospheric and stomatal resistance, and comment on any differences. The measured roughness length is 2.07 meters, and the measurement level is 45 meters. Also assume k = 0.41.

- Using the tabulated data and the Penman-Monteith approach, compute hourly transpiration. Assume that ground heat loss is negligible.

| Day | month | year | local time | Solar radiation (W m-2) | Net radiation (W m-2) | Aspirated wet bulb depression (deg C) | Air temperature (deg C) | wind speed m/sec) | wind direction | rainfall (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dec | 2007 | 6:00 | 78 | 54 | 0.7 | 24.3 | 1 | 200 | 0 |
| 1 | Dec | 2007 | 7:00 | 214 | 163 | 1.14 | 25.6 | 0.6 | 190 | 0 |
| 1 | Dec | 2007 | 8:00 | 252 | 186 | 1.48 | 26.4 | 0.7 | 155 | 0 |
| 1 | Dec | 2007 | 9:00 | 565 | 402 | 2.75 | 28 | 1.6 | 117 | 0 |
| 1 | Dec | 2007 | 10:00 | 548 | 422 | 3.81 | 29.3 | 1.3 | 123 | 0 |
| 1 | Dec | 2007 | 11:00 | 617 | 462 | 4.64 | 30.1 | 1.3 | 104 | 0 |
| 1 | Dec | 2007 | 12:00 | 378 | 289 | 4.72 | 30.2 | 1.4 | 104 | 0 |
| 1 | Dec | 2007 | 13:00 | 55 | 35 | 2.33 | 26.4 | 1.8 | 151 | 3.38 |
| 1 | Dec | 2007 | 14:00 | 83 | 57 | 1.83 | 25.7 | 1.7 | 4 | 0.38 |
| 1 | Dec | 2007 | 15:00 | 65 | 38 | 2.61 | 26.6 | 1.3 | 16 | 0 |
| 1 | Dec | 2007 | 16:00 | 15 | 3 | 1.54 | 25.9 | 1 | 119 | 0.75 |
| 1 | Dec | 2007 | 17:00 | 0 | -10 | 2.4 | 25.9 | 1.1 | 73 | 0.13 |
| 1 | Dec | 2007 | 18:00 | 0 | -16 | 3.09 | 26.4 | 1.9 | 103 | 1.12 |

Problem 2:

Use StarHydro software for this problem. It can be downloaded from the site:

http://web.mit.edu/star/hydro/

For this problem, use the map “StarHydro_Coarse”

- First, delineate a watershed with an approximate area of 10km2. Submit the image of your watershed using a screen capture.

- Next, look at the hydrograph for the delineated watershed:

  - Given V(channel) = 1 m sec-1 and Voverland/Vchannel = 1/10.

  - For rainfall intensity 12 mm hr-1 and duration 1 hour

- Now assume Voverland/Vchannel= 1/25 and recalculate hydrograph. Comment on any differences.

- Look at the pulse of rainfall events. The ratio between overland and channel velocity is 1:10. The rainfall events are:

| Duration (hr) | Rain intensity (mm hr-1) |
| --- | --- |
| 1 | 14 |
| 3 | 10 |
| 2 | 6 |
| 1 | 5 |

Comment on the hydrograph obtained from the pulse of rainfall events.